|  | Faculty of Information Science and Technology Universiti Kebangsaan Malaysia 43600 Bangi Selangor |
| --- | --- |
| TTTK2103 Computer Network Technology |  |

PACKET TRACER

2.5.5 Packet Tracer - Configure Initial Switch Settings

| Name | Matrix Number |
| --- | --- |
| Mariam binti Masuan | A182976 |
| Mohamad Saiful Nizam bin Abd Aziz | A179830 |

2.5.5 Packet Tracer - Configure Initial Switch Settings

Part 1: Verify the Default Switch Configuration

Step 1: Enter privileged EXEC mode.

Step 2: Examine the current switch configuration.

a. How many Fast Ethernet interfaces does the switch have?

Answer: 24

b.How many Gigabit Ethernet interfaces does the switch have?

Answer: 2

c.What is the range of values shown for the vty lines?

Answer: 0-15

d.Which command will display the current contents of non-volatile random-	access memory (NVRAM)?

Answer: show startup-configuration

e.Why does the switch respond with “startup-config is not present?”

Answer: Because the configuration file was not saved to NVRAM. Currently it is 	only located in RAM.

Part 2: Create a Basic Switch Configuration

Step 1: Assign a name to a switch.

Step 2: Secure access to the console line.

Why is the login command required?

Answer: In order for the password checking process to work, it requires both 	the 	login and password commands.

Step 3: Verify that console access is secured.

Step 4: Secure privileged mode access.

Step 5: Verify that privileged mode access is secure.

Step 6: Configure an encrypted password to secure access to privileged mode.

Step 7: Verify that the enable secret password is added to the configuration file.

What is displayed for the enable secret password?

Answer: $1$mERr$ILwq/b7kc.7X/ejA4Aosn0

Why is the enable secret password displayed differently from what we 		configured?

Answer: The enable secret is shown in encrypted form, whereas the enable 		password is in plain text.

Step 8: Encrypt the enable and console passwords.

If you configure any more passwords on the switch, will they be displayed in the 	configuration file as plain text or in encrypted form?

Explain. The service password-encryption command encrypts all current and future 	passwords.

Part 3: Configure a message of the day (MOTD) banner.

Step 1: Configure a message of the day (MOTD) banner.

When will this banner be displayed?

Answer: The message will be displayed when someone enters the switch through the 	console port.

Why should every switch have a MOTD banner?

Every switch should have a banner to warn unauthorized users that access is 	prohibited but can also be used for sending messages to network 	personnel/technicians (such as impending system shutdowns or who to contact for 	access).

Part 4: Save and Verify Configuration Files to NVRAM

Step 1: Verify that the configuration is accurate using the show run command.

What is the shortest, abbreviated version of the copy running-config startup-	config command?

Answer: cop r s

Which command will display the contents of NVRAM?

Answer: show startup-config

Are all the changes that were entered recorded in the file?

Answer: Yes, it is the same as the running configuration.

Part 5: Configure S2